Star topology in Cisco Packet Tracer is achieved by connecting multiple end devices (PCs, laptops) directly to a central networking device—a switch or hub—using copper straight-through cables. Each node requires a dedicated, unique IP address to enable communication through the central hub.

YouTube +4

Steps to Create a Star Topology in Packet Tracer:

- Place Components: Open Packet Tracer and drag a central device (e.g., Switch 2950-24) from "Network Devices" to the workspace.

- Add End Devices: Add several end devices (PCs) to the workspace, placing them around the central switch.

- Connect Devices: Select the "Connections" tool (lightning bolt), choose "Copper Straight-Through" cables, and connect each PC's FastEthernet0 port to an available port on the switch.

- Configure IP Addresses: Click on each PC, go to the Desktop tab, open IP Configuration, and assign static IP addresses (e.g., , etc.).

- Verify Connectivity: Open the Command Prompt on a PC and use the ping command (e.g., ping 192.168.1.2) to test connectivity to other nodes.

YouTube +4