How Star Topology Is Achieved in Cisco Packet Tracer

1. Open Cisco Packet Tracer

Launch Packet Tracer and start with a new blank workspace.

2. Add the Central Device (Switch)

At the bottom-left, click Network Devices

Choose Switches

Drag a switch  into the workspace

Note:This switch is the center of the star.

3. Add End Devices

Click End Devices

Drag multiple PCs (PC0, PC1, PC2, etc.) into the workspace

Place them around the switch (like spokes around a wheel)

This physical layout visually represents the star topology.

4. Connect Devices to the Switch

Click the Connections (lightning bolt icon)

Select Copper Straight-Through cable

Connect:

PC0 → Switch (FastEthernet0/1)

PC1 → Switch (FastEthernet0/2)

PC2 → Switch (FastEthernet0/3)

…and so on

Note:

Each PC connects only to the switch, not to other PCs — this is what makes it a star topology.

5. Configure IP Addresses (Optional but Recommended)

To allow communication:

Click on a PC

Go to Desktop → IP Configuration

Assign IPs in the same network, for example:

PC0: 192.168.1.1

PC1: 192.168.1.2

PC2: 192.168.1.3

Subnet Mask: 255.255.255.0

Repeat for each PC.

6. Test the Network

Click on a PC

Go to Desktop → Command Prompt

Use:

Copy code

ping 192.168.1.2

If the ping succeeds, communication through the central switch is working — confirming the star topology.