Title: Creating a Star Topology in Cisco Packet Tracer

Introduction: A star topology is a network configuration where all devices are connected to a central device, usually a switch. This central device acts as a hub to manage communication between devices. Star topologies are easy to manage, scalable, and isolate faults effectively.

Materials Needed:

- Cisco Packet Tracer software

- PCs or laptops

- Switch (e.g., 2960)

- Copper straight-through cables

Step-by-Step Guide:

Step 1: Open Cisco Packet Tracer

- Launch Cisco Packet Tracer.

- Open a new project (File → New).

Step 2: Select the Devices

- In the bottom-left corner, select End Devices and choose PC.

- Drag and drop 4–5 PCs onto the workspace.

- Click Network Devices → Switches → select a 2960 switch.

- Drag the switch to the center of the workspace.

Step 3: Connect the Devices to the Switch

- Click the Connections (lightning bolt icon).

- Select Copper Straight-Through cable.

- Click on the first PC → choose FastEthernet0.

- Click on the switch → choose FastEthernet0/1.

- Repeat steps 3–4 for all other PCs, using FastEthernet0/2, 0/3, etc.

Step 4: Configure the IP Addresses

- Click on a PC → Desktop → IP Configuration.

- Assign unique IP addresses and subnet masks:

  - PC1: 192.168.1.1 / 255.255.255.0

  - PC2: 192.168.1.2 / 255.255.255.0

  - PC3: 192.168.1.3 / 255.255.255.0

  - PC4: 192.168.1.4 / 255.255.255.0

- Leave the default gateway blank (unless connecting a router).

Step 5: Test the Connectivity

- On PC1, go to Desktop → Command Prompt.

- Type ping 192.168.1.2 to test connectivity with PC2.

- Repeat ping tests between all PCs to ensure successful communication.

Step 6 (Optional): Add a Router for Internet Access

- Place a router in the workspace.

- Connect the router to the switch using a straight-through cable.

- Configure the router IP as the default gateway for all PCs.

Step 7: Visualize the Star Topology

- Central switch in the middle.

- Cables radiating out to each PC.

- All PCs connected directly to the switch and not to each other.

- This completes the star topology setup.