RUBY SPRINGFIELD COLLEGE PROJECT

Q.  Using Cisco packet tracer explain how star topology is being achieved.

ANSWER

1. What is Star Topology?

A star topology is a network setup where all computers (nodes) are connected individually to a single central device, such as a Switch or a Hub. It looks like a star because all cables radiate out from the center. If one cable breaks, only that computer goes down, not the whole network.

2. Components Needed in Packet Tracer

To build this, you need the following items from the bottom-left panel:

- Central Device: 1 Switch (e.g., 2950-24) or 1 Hub (Hub-PT).

- End Devices: 4-6 PCs (Generic PCs).

- Cabling: Copper Straight-Through cable (solid black line).

3. Step-by-Step Implementation in Packet Tracer

Step 1: Place Devices

- Open Cisco Packet Tracer.

- Click on Network Devices -> Switches. Drag a 2950-24 switch to the middle of the workspace.

- Click on End Devices. Drag four PCs and place them around the switch (like rays of a star).

Step 2: Connect Devices

- Click on the Connections icon (lightning bolt).

- Select the Copper Straight-Through cable (solid black line).

- Click on a PC, select FastEthernet0.

- Click on the Switch, select an available port (e.g., FastEthernet0/1).

- Repeat this for all PCs until they are all connected to the switch.

Step 3: Configure IP Addresses

For the computers to talk to each other, they need unique addresses.

- Click on PC0. Go to the Desktop tab, then IP Configuration.

- Enter IP Address: 192.168.1.1

- Subnet Mask: 255.255.255.0 (it will fill automatically).

- Repeat for other PCs, changing the last number (e.g., 192.168.1.2, 192.168.1.3, etc.).

Step 4: Verify Connectivity (Simulation)

- Click on PC0.

- Go to the Desktop tab and open the Command Prompt.

- Type ping 192.168.1.3 (or any other PC's IP) and press Enter.

- If you see "Reply from...", the star topology is successfully configured.

4. Summary Table for Assignment

| Feature | Description in Packet Tracer |
| --- | --- |
| Central Point | Switch/Hub (2950-24) |
| End Nodes | PCs (PC0-PC3) |
| Cable Type | Copper Straight-Through |
| Key Advantage | If one PC fails, others keep working |
| Key Weakness | If the Switch fails, the whole network stops |

5. Why use a Switch?

While hubs work, a Switch is better because it is more "intelligent." When PC0 sends a message to PC2, the switch sends it directly to PC2. A hub would broadcast (send) the message to everyone, causing a lot of traffic.