Star Topology Using Cisco Packet Tracer

1. Introduction to Star Topology

A Star Topology is a network layout in which all end devices are directly connected to a central device, usually a switch or hub. Communication between devices always passes through this central device.

In Cisco Packet Tracer, a star topology is created by connecting multiple PCs to a single switch using straight-through Ethernet cables.

2. Components Required in Cisco Packet Tracer

To achieve a star topology in Cisco Packet Tracer, the following devices are. used:Switch (e.g., Cisco 2960)End Devices (PCs, laptops, printers, servers)

Copper Straight-

Through Cables

Why a Switch?

It acts as the central connection point

It forwards data only to the intended device using MAC addresses

It reduces network collusion

3. Diagram of Star Topology

Logical Diagram

PC1

|

|

PC2 ---- Switch ---- PC3

|

|

PC4

The switch is at the center

Each PC has a dedicated link to the switch

No direct PC-to-PC connection

4. Steps to Create Star Topology in Cisco Packet Tracer

Step 1: Open Cisco Packet Tracer

Launch the application

Select a new blank workspace

Step 2: Add Network Devices

From Network Devices, choose Switch

Drag a Cisco 2960 switch to the workspace

From End Devices, add multiple PCs

Step 3: Connect Devices

Select Copper Straight-Through cable

Connect:

PC FastEthernet0 → Switch FastEthernet0/1

PC FastEthernet0 → Switch FastEthernet0/2

Repeat for all PCs

Physical Diagram in Packet Tracer

PC1 ──────┐

PC2 ──────┼── Switch (2960)

PC3 ──────┤

PC4 ──────┘

5. IP Address Configuration

To enable communication:

Click on each PC

Go to Desktop → IP Configuration

Assign IP addresses in the same network

Example:

Device	IP Address	Subnet Mask

PC1	192.168.1.1	255.255.255.0

PC2	192.168.1.2	255.255.255.0

PC3	192.168.1.3	255.255.255.0

No default gateway is needed for a simple LAN.

6. How Communication Works in Star Topology

PC1 sends data to PC3

Data first reaches the switch

The switch checks its MAC address table

Data is forwarded only to PC3’s port

This makes star topology:

Efficient

Secure

Fast

7. Advantages of Star Topology

Easy to install and manage

Failure of one PC does not affect others

Easy to add or remove devices

Better performance due to dedicated links

8. Disadvantages of Star Topology

Failure of the central switch brings down the entire network

Requires more cabling

Slightly higher cost compared to bus topology

9. Conclusion

In Cisco Packet Tracer, a star topology is achieved by connecting multiple end devices to a central switch using straight-through cables. The switch controls data flow, improves performance, and ensures reliable communication. Due to its simplicity and efficiency, star topology is widely used in LANs, schools, offices, and enterprises.

If you want, I can also:

Convert this into handwritten-style exam notes

Add Packet Tracer screenshots explanation

Shorten it further for 5-mark or 10-mark answers