Using Cisco packet tracer explain with step how star topology is been achieved

Alright 👍 let’s walk through it step by step in a clear, exam-friendly way.

What is a Star Topology?

A star topology is a network layout where all devices are connected to one central device (usually a switch).

All communication passes through this central device.

In Cisco Packet Tracer, we achieve star topology by connecting multiple PCs to one switch.

Steps to Achieve Star Topology Using Cisco Packet Tracer

Step 1: Open Cisco Packet Tracer

Launch Cisco Packet Tracer

You’ll see an empty workspace (Logical view)

Step 2: Add the Central Device (Switch)

Click Network Devices (bottom left)

Select Switches

Choose 2960 Switch

Drag and drop it into the workspace

👉 This switch is the center of the star

Step 3: Add End Devices (PCs)

Click End Devices

Select PC

Drag at least 3 or 4 PCs into the workspace

👉 These are the “points” of the star

Step 4: Connect Devices to the Switch

Click Connections (lightning icon)

Select Copper Straight-Through cable

Connect:

PC1 → Switch (FastEthernet 0/1)

PC2 → Switch (FastEthernet 0/2)

PC3 → Switch (FastEthernet 0/3)

Continue for other PCs

⚠️ Important:

Each PC must connect directly to the switch

PCs are not connected to each other

Step 5: Assign IP Addresses to PCs

For each PC:

Click the PC

Go to Desktop

Click IP Configuration

Assign IP addresses like this:

PC

IP Address

Subnet Mask

PC1

192.168.1.1

255.255.255.0

PC2

192.168.1.2

255.255.255.0

PC3

192.168.1.3

255.255.255.0

(Default Gateway can be left empty)

Step 6: Test the Network

Click PC1

Go to Desktop → Command Prompt

Type:

Copy code

ping 192.168.1.2

Repeat for other PCs

✔️ If replies are successful, the network is working.

How This Forms a Star Topology

The switch is the center

All PCs connect directly to the switch

If one PC fails, others still work

If the switch fails, the whole network goes down

This structure clearly shows a star shape.

Diagram Description (for exams)

“In a star topology, all nodes are connected to a central switch. Communication between devices passes through the switch.”

If you want, I can also:

Draw a simple labeled diagram

Explain advantages and disadvantages

Explain star vs bus vs ring

Help you write this as a WAEC/NECO exam answer

Just tell me 😊Alright 👍 let’s walk through it step by step in a clear, exam-friendly way.

What is a Star Topology?

A star topology is a network layout where all devices are connected to one central device (usually a switch).

All communication passes through this central device.

In Cisco Packet Tracer, we achieve star topology by connecting multiple PCs to one switch.

Steps to Achieve Star Topology Using Cisco Packet Tracer

Step 1: Open Cisco Packet Tracer

Launch Cisco Packet Tracer

You’ll see an empty workspace (Logical view)

Step 2: Add the Central Device (Switch)

Click Network Devices (bottom left)

Select Switches

Choose 2960 Switch

Drag and drop it into the workspace

👉 This switch is the center of the star

Step 3: Add End Devices (PCs)

Click End Devices

Select PC

Drag at least 3 or 4 PCs into the workspace

👉 These are the “points” of the star

Step 4: Connect Devices to the Switch

Click Connections (lightning icon)

Select Copper Straight-Through cable

Connect:

PC1 → Switch (FastEthernet 0/1)

PC2 → Switch (FastEthernet 0/2)

PC3 → Switch (FastEthernet 0/3)

Continue for other PCs

⚠️ Important:

Each PC must connect directly to the switch

PCs are not connected to each other

Step 5: Assign IP Addresses to PCs

For each PC:

Click the PC

Go to Desktop

Click IP Configuration

Assign IP addresses like this:

PC

IP Address

Subnet Mask

PC1

192.168.1.1

255.255.255.0

PC2

192.168.1.2

255.255.255.0

PC3

192.168.1.3

255.255.255.0

(Default Gateway can be left empty)

Step 6: Test the Network

Click PC1

Go to Desktop → Command Prompt

Type:

Copy code

ping 192.168.1.2

Repeat for other PCs

✔️ If replies are successful, the network is working.

How This Forms a Star Topology

The switch is the center

All PCs connect directly to the switch

If one PC fails, others still work

If the switch fails, the whole network goes down

This structure clearly shows a star shape.

Diagram Description (for exams)

“In a star topology, all nodes are connected to a central switch. Communication between devices passes through the switch.”