Rachel Michael

Serial number 34,

JSS3A

ASSIGNMENT

Using cisco packet tracer, explain with steps and an aid of diagram, how star topology is achieved.

## Step-by-Step Procedure in Cisco Packet Tracer

### Step 1: Open Cisco Packet Tracer

- Launch Cisco Packet Tracer

- You’ll see the workspace (Logical view by default)

### Step 2: Add the Central Switch

- Click Network Devices (bottom-left)

- Select Switches

- Drag a 2960 switch into the workspace

This switch is the center of the star

### Step 3: Add End Devices (PCs)

- Click End Devices

- Drag PCs (PC0, PC1, PC2, etc.) into the workspace

- Place them around the switch

### Step 4: Connect Devices to the Switch

- Click Connections (lightning icon)

- Choose Copper Straight-Through

- Connect:

  - PC → FastEthernet0

  - Switch → FastEthernet0/1, 0/2, 0/3, etc.

- Repeat for all PCs

- Wait a few seconds — links will turn green, meaning they’re active.

### Step 5: (Optional) Assign IP Addresses

For communication testing:

- Click a PC

- Go to Desktop → IP Configuration

- Assign IPs in the same network, e.g.

| PC | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 |
| PC1 | 192.168.1.2 | 255.255.255.0 |
| PC2 | 192.168.1.3 | 255.255.255.0 |

### Step 6: Test Connectivity

- Open Command Prompt on a PC

- Use:

ping 192.168.1.2

✔️ Successful replies confirm the star topology works.

## 4. Diagram of Star Topology

Here’s a simple logical diagram:

PC0

|

|

PC1 —— Switch —— PC2

|

|

PC3

Explanation:

- The switch is the central node

- All PCs connect directly to the switch

- No PC connects to another PC directly

## 5. Why This Is a Star Topology

✔ Single central device (switch)
✔ All communication passes through the center
✔ Failure of one PC link does not affect others