STAR TOPOLOGY

In Cisco Packet Tracer, a Star Topology is a network layout where every end device (PC, printer, etc.) is connected to a single central node, typically a Switch or a Hub.

Steps to Achieve Star Topology

- Select a Central Device:

- Navigate to Network Devices > Switches.D

- rag and drop a switch (e.g., 2960) into the center of your workspace. A switch is preferred over a hub because it reduces data collisions and improves performance.

- Add End Devices:

- Go to End Devices and select PC or Laptop.

- Place several devices (e.g., 4 to 6) in a circle around the central switch to visualize the "star" pattern.

- Establish Connections:

- Go to Connections (lightning bolt icon).

- Select the Copper Straight-Through cable.

- Click on a PC (select FastEthernet0), then click on the Switch and choose any available port (e.g., FastEthernet0/1).

- Repeat this for all PCs until they are all connected to the central switch.

- Configure IP Addresses:

- Click on each PC, go to the Desktop tab, and open IP Configuration.

- Assign a static IP address to each device within the same subnet.

- PC0: 192.168.1.1

- PC1: 192.168.1.2

- Subnet Mask: 255.255.255.0 (for all).

- Test Connectivity:

- Wait for the link lights to turn from amber/red to green.

- Open the Command Prompt on one PC and type ping 192.168.1.2 (the IP of another PC) to verify communication.

Key Characteristics & Benefits

- Centralized Communication: All data must pass through the central switch before reaching its destination.

- Fault Isolation: If one cable or PC fails, only that specific connection is lost; the rest of the network remains fully operational.

- Scalability: Adding a new device is as simple as plugging another cable into the switch without disrupting existing users.

- Single Point of Failure: The main disadvantage is that if the central switch fails, the entire network goes down.