# BENJAMINE AUGUSTINE

# Using Cisco Packet Tracer to Build and Achieve a Network Topology

## Introduction

Cisco Packet Tracer is a networking simulation software developed by Cisco. It is used to design, configure, and test network topologies without using physical networking devices. It helps students understand how computers and network devices communicate with one another.

## Steps to Start and Achieve a Network Topology Using Cisco Packet Tracer

### Step 1: Open Cisco Packet Tracer

- Launch the Cisco Packet Tracer application.

- Click on File → New to open a blank workspace.

### Step 2: Add Network Devices

- From the bottom-left corner, select End Devices and drag at least one PC into the workspace.

- Select Network Devices and drag a Switch and a Router into the workspace.

- Arrange the devices neatly on the screen.
This arrangement is called the network topology.

### Step 3: Connect the Devices

- Click on the Connections (lightning icon).

- Choose Copper Straight-Through cable.

- Connect:

  - PC to Switch

  - Switch to Router

- After connecting, wait a few seconds for the lights to turn green.

### Step 4: Assign IP Addresses

#### On the PC

- Click on the PC.

- Go to Desktop → IP Configuration.

- Enter:

  - IP Address: 192.168.1.2

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.1

#### On the Router

- Click on the router and open CLI.

- Type the following commands:

enable

configure terminal

interface gigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

### Step 5: Test the Network

- Click on the PC.

- Open Desktop → Command Prompt.

- Type:

ping 192.168.1.1

If a reply is received, it means the network is working correctly.

## How to Know the Topology Is Achieved

A network topology is said to be achieved when:

- All cables show green lights.

- IP addresses are correctly assigned.

- Devices can successfully communicate using the ping command.

## Conclusion

Cisco Packet Tracer makes it easy to design and test network topologies. By adding devices, connecting them, assigning IP addresses, and testing communication, a simple network topology can be successfully achieved. This helps students understand basic networking concepts in a practical way.